44:02:07:74.  Storage areas for refuse, recyclables, and returnables. A storage area for refuse, recyclables, and returnables must meet the requirements specified:

(1)  An inside storage room or area, an outside storage area or enclosure, and individual receptacles must be of sufficient number and capacity to hold refuse, recyclables, and returnables that accumulate;

(2)  The floors, floor coverings, walls, wall coverings, and ceilings of an inside storage area must be designed and constructed so they are smooth, nonabsorbent, and easily cleanable;

(3)  An outdoor storage surface for refuse, recyclables, and returnables must be constructed of nonabsorbent material such as concrete or asphalt and must be smooth, durable, and sloped to drain;

(4)  If used, an outdoor enclosure for refuse, recyclables, and returnables must be constructed of durable and cleanable materials;

(5)  Storage areas, enclosures, and receptacles for refuse, recyclables, and returnables must be maintained in good repair;

(6)  Refuse, recyclables, and returnables must be stored in equipment, refuse receptacles, storage areas, or enclosures so that they are inaccessible to insects and rodents;

(7)  Refuse receptacles not meeting the requirements specified in § 44:02:07:73, such as receptacles that are not rodent-resistant, unprotected plastic bags and paper bags, or baled units that contain materials with food residue, may not be stored outside; and

(8)  The location of equipment, receptacles, storage areas, or enclosures for refuse, recyclables, and returnables may not create a public health nuisance.

Source: 23 SDR 195, effective May 26, 1997.

General Authority: SDCL 34-1-17, 34-18-22.

Law Implemented: SDCL 34-18-22, 34-18-25.