44:03:01:06.01.  Exemption from radiation licensing. Electronic equipment that produces radiation incidental to its operation for other purposes is exempt from § 44:03:01:06 if the radiation dose equivalent rate averaged over an area of ten square centimeters does not exceed 0.5 millirem or 0.005 millisievert per hour at five centimeters from any accessible surface of the equipment. Domestic television receivers and cathode ray tube monitors and televisions are exempt from the licensing requirement in § 44:03:01:06.

Source: SL 1975, ch 16, § 1; transferred from § 44:03:01:03, 6 SDR 93, effective July 1, 1980; 26 SDR 96, effective January 23, 2000; 50 SDR 41, effective October 11, 2023.

General Authority: SDCL 34-21-4.1, 34-21-15.

Law Implemented: SDCL 34-21-20.