61:15:06:06.  Floor covering requirements. Interior floor finish materials in a group family day care home shall be a Class 2 material in required exits, exit passageways, or corridors providing exit access. A Class 2 material is any material having a minimum critical radiant flux of 0.22 watts per square centimeter as established by the procedures specified in NFPA-253, "Standard Method of Test for Critical Radiant Flux of Floor Covering Systems Using a Radiant Heat Energy Source," 2015 edition. If an automatic sprinkler system is installed, floor covering materials need not be classified. Floor covering requirements apply to new construction and to replacing of floor covering in an existing facility.

Source: 18 SDR 107, effective January 1, 1992; 23 SDR 32, effective September 11, 1996; 33 SDR 113, effective January 2, 2007; 37 SDR 235, effective June 28, 2011; 43 SDR 56, effective October 17, 2016.

General Authority: SDCL 34-29B-2.

Law Implemented: SDCL 34-29B-2.

Reference: NFPA 253, "Standard Method of Test for Critical Radiant Flux of Floor Covering Systems Using a Radiant Heat Energy Source," 2015 edition, National Fire Protection Association. Copies may be obtained from the National Fire Protection Association, 1 Batterymarch Park, Quincy, Massachusetts 02169-9101; $35.55 for members and $39.50 for nonmembers. Telephone: 1-617-770-3000.