DEPARTMENT OF TRANSPORATION

ACCESS MANAGEMENT

SOUTH DAKOTA ACCESS-LOCATION CRITERIA

Chapter 70:09:02

Appendix A

SEE: 70:09:02:01

Source: 29 SDR 66, effective November 18, 2002; 29 SDR 177, effective July 1, 2003.

South Dakota Access-Location Criteria

| ## Access Class |  |  | Signal Spacing Distance (mile) | Median Opening Spacing (mile) | #### Minimum Unsignalized Access Spacing (feet) | Access Density | Denial of Direct Access When Other Available |
| --- | --- | --- | --- | --- | --- | --- | --- |
| # Interstate |  |  | N/A | N/A | N/A | N/A | Yes |
| # Expressway |  |  | ½ | ½ | 2640 | at half-mile increments | Yes |
| Free Flow Urban |  |  | ½ | ½ F, ¼ D | 1320 | at quarter-mile increments | Yes |
| Intermediate Urban |  |  | ½ | ½ F, ¼ D | 660 | at eighth-mile increments | Yes |
| Urban Developed |  |  | ¼ | ¼ | 100 | 2 accesses/block face | Yes |
| Urban Fringe |  |  | ¼ | ½ F, ¼ D | 1000 | 5 accesses/side/mile | Yes |
| Rural |  |  | N/A | N/A | 1000 | 5 accesses/side/mile | Yes |
| NOTES: |  |  |  |  |  |  |  |
|  | 1. | Access to the Interstate system is governed by SDDOT interchange policy. No new |  |  |  |  |  |
|  |  | access shall be provided on non-interstate routes within 1/8 mile of interstate ramp |  |  |  |  |  |
|  |  | terminals. |  |  |  |  |  |
|  | 2. | N/A = Not Applicable, F = Full Movement – all turns and through movements |  |  |  |  |  |
|  |  | provided, D = Directional Only – certain turning and through movements not |  |  |  |  |  |
|  |  | provided. |  |  |  |  |  |
|  | 3. | SDDOT may defer to stricter local standards. |  |  |  |  |  |
|  | 4. | SDDOT will seek opportunities to reduce access density wherever possible. |  |  |  |  |  |
|  | 5. | Rural class minimum unsignalized access spacing may be reduced to 660' by the |  |  |  |  |  |
|  |  | Area Engineer, based on results of an engineering study as described in § 70:09:02:01. |  |  |  |  |  |

Access Class Definitions

# Interstate – the designated Interstate highway system, including I-90, I-29, I-229, and I-190.

# Expressway – high-speed divided highways serving interstate and regional travel needs.

# Free Flow Urban – higher speed facilities with access subordinate to through traffic movement.

# Intermediate Urban – serves through traffic while allowing moderate access density.

# Urban Developed – traffic artery with high access density. Access and through movement have equal priority.

# Urban Fringe – rural highway serving developing area immediately adjacent to a city or town. Access regulated to provide future through-traffic priority.

# Rural – low volume, high-speed facility. Access points are spaced for safety and operations efficiency.