74:31:03:03.  Abatement project designer training course requirements. The abatement project designer training course must be at least three days long and must include lectures, demonstrations, a field trip, course review, and a written examination. The following topics must be included:

(1)  Background information on asbestos:

(a)  Identification of asbestos;

(b)  Examples and discussions of the uses and locations of asbestos in buildings;

(c)  Physical appearance of asbestos;

(2)  Potential health effects related to asbestos exposure:

(a)  The nature of asbestos-related diseases;

(b)  Routes of exposure;

(c)  Dose-response relationships and the lack of a safe exposure level;

(d)  The synergistic effect between cigarette smoking and asbestos exposure;

(e)  The latency period of asbestos-related diseases;

(f)  A discussion of the relationship between asbestos exposure and asbestosis, lung cancer, mesothelioma, and cancer of other organs;

(3)  Overview of abatement construction projects:

(a)  Abatement as a portion of a renovation project;

(b)  OSHA requirements for notification of other contractors on a multiemployer site in 29 C.F.R. § 1926.58 (July 1, 1993) as amended by 59 Fed. Reg. 40,964 to 41,162, inclusive (August 10, 1994);

(4)  Safety system design specifications:

(a)	Design, construction, and maintenance of containment barriers and decontamination enclosure systems;

(b)	Positioning of warning signs;

(c)	Electrical and ventilation system lockout;

(d)	Working techniques for minimizing fiber release;

(e)	Entry and exit procedures for the work area;

(f)	Use of wet methods;

(g)	Use of negative pressure exhaust ventilation equipment;

(h)	Use of high efficiency particulate aerosol (HEPA) vacuums;

(i)	Clean-up and disposal of asbestos;

(j)	Work practices as they apply to encapsulation, enclosure, and repair;

(k)	Use of glove bags and a demonstration of glove bag use;

(l)	Proper techniques for initial cleaning;

(5)  Field trip:

(a)  A visit to an abatement site or other suitable building site. For example, a metal pole building without any HVAC systems would not be considered a suitable site. The visit must include:

(i)		On-site discussions of abatement design;

(ii)	A building walk-through inspection;

(iii)	A discussion of rationale for the concept of functional spaces during the walk-through;

(6)  Employee personal protective equipment:

(a)	Classes and characteristics of respirators;

(b)	Limitations of respirators;

(c)	Selection, inspection, donning, use, maintenance, and storage procedures;

(d)	Methods for field testing of the facepiece-to-face seal (positive and negative pressure fitting tests);

(e)	Qualitative and quantitative fit testing procedures;

(f)	Variability between field and laboratory protection factors;

(g)	Factors that alter respirator fit, for example, facial hair;

(h)	Components of a respiratory protection program;

(i)	Selection and use of personal protective clothing;

(j)	Use, storage, and handling of nondisposable clothing;

(k)	Regulations found in subdivision (18) of this section covering personal protective equipment;

(7)  Additional safety hazards:

(a)  Hazards encountered during abatement activities and how to deal with them, including:

(i)		Electrical hazards;

(ii)	Heat stress;

(iii)	Air contaminants other than asbestos;

(iv)	Fire and explosion hazards;

(8)  Fiber aerodynamics and control:

(a)  Aerodynamic characteristics of asbestos fibers;

(b)  Importance of containment barriers;

(c)  Settling time for asbestos fibers;

(d)  Wet methods in abatement;

(e)  Aggressive air monitoring following abatement;

(f)  Aggressive air movement and negative pressure exhaust ventilation as a clean-up method;

(9)  Designing abatement solutions:

(a)  Discussions of removal, enclosure, and encapsulation methods;

(b)  Asbestos waste disposal;

(10)  Final clearance process:

(a)  Discussion of the need for a written sampling rationale for aggressive final air clearance;

(b)  Requirements of a complete visual inspection;

(c)  Discussion of the relationship of visual inspection to final air clearance;

(11)  Budgeting and cost estimation:

(a)  Development of cost estimates;

(b)  Present costs of abatement versus future operations and maintenance costs;

(c)  Setting priorities for abatement jobs to reduce cost;

(12) Writing abatement specifications:

(a)  Means and methods specifications versus performance specifications;

(b)  Design of abatement in occupied buildings;

(c)  Modification of guide specifications to a particular building;

(d)  Worker and building occupant health and medical considerations;

(e)  Replacement of ACM with a substitute for asbestos;

(f)  Preparation of and need for a written project design;

(13)  Preparing abatement drawings:

(a)  Use of as-built drawings as base drawings;

(b)  Use of inspection photographs and on-site reports;

(c)  Particular problems in abatement drawings;

(d)  Significance and need for drawings;

(e)  Methods of preparing abatement drawings;

(f)  Diagramming containment barriers;

(g)  Relationship of drawings to design specifications;

(14)  Contract preparation and administration;

(15)  Legal liabilities and defenses:

(a)  Insurance considerations;

(b)  Bonding;

(c)  Hold harmless clauses;

(d)  Use of abatement contractor's liability insurance;

(e)  Claims made versus occurrence policies;

(16)  Replacement of asbestos with asbestos-free substitutes;

(17)  Role of other consultants:

(a)  Development of technical specification sections by industrial hygienists or engineers;

(b)  The multidisciplinary team approach to abatement design;

(18)  Occupied buildings:

(a)  Special design procedures required in occupied buildings;

(b)  Education of occupants;

(c)  Extra monitoring recommendations;

(d)  Staging of work to minimize exposure to occupants;

(e)  Scheduling of renovation to minimize exposure;

(19)  Regulatory review -- procedures and standards:

(a)  Requirements of AHERA;

(b)  National emission standard for hazardous air pollutants in 40 C.F.R. § 61, Subparts A (general provisions) and M (national emission standard for asbestos) (July 1, 1993);

(c)  OSHA standards for permissible exposure to airborne concentrations of asbestos fibers and respiratory protection in 29 C.F.R. § 1910.134 (July 1, 1993) as amended by 59 Fed. Reg. 40,964 to 41,162, inclusive (August 10, 1994);

(d)  EPA worker protection rule in 40 C.F.R. § 763, Subpart G (July 1, 1993);

(e)  OSHA asbestos construction standard in 29 C.F.R. § 1926.58 (July 1, 1993) as amended by 59 Fed. Reg. 40,964 to 41,162, inclusive (August 10, 1994);

(f)  OSHA hazard communication standard in 29 C.F.R. § 1926.59 (July 1, 1993);

(20) Course review -- a review of key aspects of the training course.

Source: 14 SDR 164, effective June 5, 1988; 18 SDR 95, effective December 2, 1991; 21 SDR 101, effective December 4, 1994.

General Authority: SDCL 34-44-3, 34-44-19.

Law Implemented: SDCL 34-44-3, 34-44-4, 34-44-20, 34-44-21.