74:54:02:17.  Perimeter of operational pollution (POP). The board may issue a water quality variance permit for a perimeter of operational pollution (POP) which meets the requirements of this chapter within which groundwater degradation is allowed only if groundwater quality standards of §§ 74:54:01:04 and 74:54:01:05 are met at the compliance monitoring point. The maximum aerial extent of the POP must be the shortest distance of the following:

(1)  Not more than a one-quarter mile radius from the point of application without approval by the board;

(2)  Not beyond the property boundaries of the permitted facility unless consent from the adjacent landowner(s) is obtained in writing and submitted as part of the application;

(3)  Not intercepting the radius of influence of any beneficial use water supply well.

The board may adjust the POP depending upon the hydrology, type of contaminants, and other factors that may affect the groundwater quality. These zones may allow for natural attenuation of contaminants to environmentally acceptable levels. A POP may not be allowed where existing natural processes may be unable to attenuate contaminants which may pose a risk to human health or the environment and where contaminants may not be contained within the POP.

Source: 14 SDR 86, effective December 24, 1987; 18 SDR 128, effective February 11, 1992; transferred from § 74:03:16:16, July 1, 1996; 31 SDR 62, effective November 7, 2004.

General Authority: SDCL 34A-2-28, 34A-2-30.

Law Implemented: SDCL 34A-2-24, 34A-2-25.