44:76:11:15.  Floor surface finish. Floors shall be easily cleanable and shall have the wear resistance appropriate for the location involved. Floors in kitchens and related spaces shall be water-resistant. In all areas where floors are subject to wetting, they shall have a nonslip finish. Adjacent dissimilar floor materials shall be flush with each other to provide a level floor surface.

Source: 42 SDR 51, effective October 13, 2015.

General Authority: SDCL 34-12-13(1) and (3).

Law Implemented: SDCL 34-12-13(1) and (3).