44:82:03:03.  Exits. There shall be two exits remote from each other on each level used for the care of residents. Each of these exits shall provide unobstructed access to the outside. One of these exits shall be a door or stairway which leads to the outside of the building at ground level. The other exit may be a window if it meets the following requirements:

(1)  It can be easily opened from the inside without the use of tools;

(2)  It provides a clear opening with a minimum dimension of at least 20 inches in width by 24 inches in height, is at least five and seven-tenths square feet in area, with a sill at 44 inches, and provides a safe means of escape; and

(3)  If a storm window, screen, or burglar guard is used, it has a quick-opening device which can be easily opened from the inside.

A ladder, folding stairs, or trap door may not be the only access to a bedroom.

Source: 44 SDR 66, effective October 17, 2017.

General Authority: SDCL 34-12-13(3).

Law Implemented: SDCL 34-12-13(3).