20:44:22:27.  Residential housing units moved to new locations. Single, modular, or multifamily residential units moved from one location to another must have at least a 100-ampere service at the new location and must meet the ground fault circuit interrupter protection, tamper resistant receptacle protection, arc-fault circuit interrupter protection, receptacle for heating and cooling equipment, weather resistant receptables and in use cover, and emergency disconnect requirements of the National Electrical Code, Article:

(1)  210.8 - ground fault circuit interrupter protection;

(2)  210.12 - arc fault circuit interrupt protection;

(3)  406.12 - tamper resistant receptacles;

(4)  210.63 - receptacle within 25 feet of the heating and ac equipment;

(5)  406.9 - weather resistant receptacles and in use covers; and

(6)  230.85 - emergency disconnects.

Source: 19 SDR 155, effective April 14, 1993; 20 SDR 222, effective July 6, 1994; transferred from § 20:44:05:32, August 12, 1994; 23 SDR 2, effective July 16, 1996; 29 SDR 87, effective December 24, 2002; 40 SDR 198, effective May 28, 2014; 43 SDR 181, effective July 10, 2017; 48 SDR 23, effective September 15, 2021.

General Authority: SDCL 36-16-12, 36-16-27.

Law Implemented: SDCL 36-16-27.